LiftMaster

SECTION 08 71 23 – COMMERICAL DOOR OPERATORS

GENERAL

Closeout Submittals:

Operation and Maintenance Data.

QUALITY ASSURANCE

Installer Qualifications: Firm specializing in work of this Section, with minimum [2] years’ experience.

WARRANTY

Manufacturer’s [2] year warranty against material and manufacturing defects.

PRODUCTS

MANUFACTURERS

LiftMaster. www.LiftMaster.com

ELECTRIC DOOR OPERATORS

Industrial Duty Operator: LiftMaster MAXUM JDC Jackshaft DC Operator, slow start, slow stop DC motor with overload protection and battery backup standard for emergency operation.

- 1.		Electric Operator: Industrial-duty assembly, cULus listed and cULus labeled, complete with DC electric motor and factory-prewired motor controls, internal auxiliary reversal circuit, floor level 3-button wall controller including open/close/stop control station with 4-line LCD display, conduit-encased wiring from control circuit to motor, and accessories required for proper operation; operator shall be capable of driving door at a speed of approximately 8 to 12 inches per second.
a.	Drive Reduction: 65:1 gearbox with #50 chain output on 700 series operators.

- b.	Brake: Standard dynamic braking from the motor on 700 series operators capable of stopping and holding a door at any position.

- c.	Electronic Limits: Fully adjustable from the wall control menu. Options to set the open and close limit in any increments to allow for very precise limit settings. All limit settings are stored in the master board at the power head and remain if the unit restarts. Factory reset or

- changing main board is the only way the system will lose the programmed limit settings.

- d.	Electric Motor: 78% High efficiency quiet DC motor that runs at 5000 RPM with a reduced

- profile of 2.7 inches OD and only 7 lbs (½ HP equivalent). One model number is field selectable to meet requirements for incoming voltage matching single phase 120V or 240V, or 3 phase 240V. 480V is also available with a unique model number for only 480V applications. 208V and 600V can be achieved with an external step-down transformer accessory.

- e.	Gearbox: Custom aluminum gearbox with upgraded bearing system. Integrated mechanical

- structure providing high strength and low vibration. Integrated mounting for encoder system

- eliminating chains and sprockets.  Custom worm design with high grade carbonized material.

- Seal Design is a high-grade Viton material with double seal.

- f.	Wall Controller: 2-wire non-polarity-sensitive floor level wall controller for easy operator/door

- commissioning and setup. 2 wires connect to WALL CTL inputs dedicated on the main board

- for easy wiring.  Limit setup, Door Motion control, Operation modes: C2, B2, B2/T, B2/TS,

- D1, E2 and FSTS. Programming connectivity (Wi-Fi, remotes, myQ), cycle counter and

- maintenance cycle counter. Programmable timer-to-close with timer defeat option, mid-stop

- programming capabilities and a max run timer to provide motor overrun protection (based on

- cycles per hour). Wall Controller is housed in a NEMA 1 enclosure and provides LCD display

- of information including diagnostics and system information for ease of troubleshooting.

- g.	Battery Backup Standard: Operators are equipped with the electronics to hook up (2) 12V

- batteries for a minimum of 20 cycles on battery backup.  Battery kit included is DC45AH, which includes battery enclosure and (2) 4.5 Ah batteries. Battery enclosure can be secured directly onto the operator chassis or can be mounted near the operator for flexibility of applications. Battery backup door control can be setup to open or close upon AC failure and Low Battery. Setting must be chosen in the menu depending on customer requirements.

- h.	Input terminals for 3 monitored safety entrapment protection devices. Can be programmed to

- protect the close or the open direction. Defaults to protect the close, but can be changed in

- the menu.

- i.	Programmable Inputs: 4 programmable inputs for dry contact accessories including non-

- monitored entrapment protection devices, car dealer input, timer defeat, all fly, truck presence

- sensors, dock sensors.

- j.	Cable Tension Monitor inputs are available and optional for installation.

- k.	Wi-Fi Connectivity: Wi-Fi-enabled operator with built-in Wi-Fi chip to access over-the-air updates for system upgrades. Operator must be connected to Wi-Fi to receive updates.

- l.	myQ Technology: 900 MHz and Wi-Fi, LiftMaster 828LM Internet Gateway, enable you to

- monitor and control door operators via smartphone, tablet, or computer.  You can connect to

- the myQ residential app or myQ Facility, which provide different features depending on your

- application need.

- m.	Standard Duty Cycle: 20 cycles per hour/90 cycles per day.

- .

- 2.	Primary Entrapment Protection Devices:

- a.	CPS-U Monitored Photo Eyes included with the operator. Other monitored photo eyes

- available including LMTBUL (Through Beam), LMRRUL (Retro Reflective), CPS-UN4, CPS-

- RPEN4, CPS-OPEN4

- b.	Monitored Wireless Edge Kit (LMWEKITU) and Monitored Wireless Edge Transmitter 			(LMWETXU)

- c.	Monitored Optical Edge System (OES-SD16) for sectional doors and (OESRD16) for rolling

- steel doors.

- d.	ASO Monitored 4-wire and 2-wire Sentir Series Edges

- e.	Monitored Light Curtains (LM36M)

- 3.	Accessories

- a.	Auxiliary Relay Board (AUXREL) for auxiliary limit switches, for switching on/off auxiliary

- devices such as lights, bells, horns/strobes and other warning devices

- b.	Pluggable Loop Detector Board (LMEXP) for use with LOOPDETLM for power-efficient

- maximum cycles on battery backup.

- c.	Chain Tensioner (CHAINTENSDC) to easily adjust the drive chain to achieve proper tension

- d.	Chain Guard (CHNGUARD) for applications under 8ft.

- e.	Front of Hood Bracket (FOHMODDC) for rolling door applications with front-of-hood mounting

- f.	HOISTMOUNTMODDC, for mounting new MAXUM J and H operators to existing L5 GH

- installations. Can be used as standalone or attach to current L5 bracket.

EXECUTION

INSTALLATION

Install in accordance with manufacturer’s instructions.

CLOSEOUT ACTIVITIES

Test and adjust operators for proper operation.

Demonstration: Demonstrate operation and programming of operators to Owner.

END OF SECTION